國立中山大學化學系

陳軍互老師實驗室

X 光粉末繞射儀收費標準暨使用規範

104年 8月 1日起實施

儀器設備說明

* 廠牌及型號：德國, Bruker D2 Phaser。

* 規格：

(1) 銅鈀 (1.54184 × 10-10 m)

(2) 燈源：300 W (30 kV, 10 mA)。

* 性能：測量粉末與多晶材料之X 光繞射。

服務項目

* 粉末或薄膜樣品之X 光廣角繞射圖。

樣品準備須知

* 粉末樣品必須以杵與研缽磨細，委託操作者樣品最少需要0.2 g。

* 薄膜樣品或固體基板之試片，面積不可大於1.5×1.5 cm，高度低於1 mm。基材厚度不同及基材本身訊號由使用者自行斟酌。

* 不接受具揮發性、毒性之樣品。

使用規則

* 一般樣品最低起始掃描範圍為 2θ = 5°

* 填寫預約表格時請自行將機台冷卻時間併入。

* 請提前預約，不接受當天預約。

* 使用完後務必填寫使用紀錄簿。

* 若自行操作者不當操作儀器，將會被停止使用權兩周，細項如下：

1. 樣品盤應以清水及酒精清洗，如果誤用其它溶劑，停權一週

2. 在未關閉 X 光燈源時就打開閘門，為嚴重安全疏失，停權兩週

3. 摔落狹縫或未以正確方式收納，停權兩週

4. 以上，累積違規達兩次者須重新認證

收費標準

| 項目 | 價格 (元) | 備註 |
| --- | --- | --- |
| 認證 | 1500 | 一間實驗室限兩人認證（自行操作） 認證三階段: (皆由儀器管理人負責) 一：教學 二：實際操作 三：考試 |

| 項目 |  | 標準收費 (元) | 備註 |
| --- | --- | --- | --- |
| 委託操作 | 校內 | 600/個 樣品 | 1.標準參數：2θ = 5° ~ 85°, increment=0.06, 約24分鐘，視使用者需要可調整，29分鐘以內皆適用標準收費 2.如未告知操作條件，一律使用標準參數。如有特殊需求須超過29分鐘者，請參考超時收費標準。 3.收費不含資料庫使用。 |
|  | 校外 | 1000/個 樣品 |  |

超時收費標準：

- 30分鐘/個樣品：單個樣品花費30~39分鐘者，「標準收費」×1.5倍。

- 40分鐘/個樣品：單個樣品花費40~49分鐘者，「標準收費」×2倍。

- 不接受50分鐘以上之樣品掃描。

機台冷卻時間規定：

1. 20分鐘/個樣品：冷卻 10 分鐘

2. 30分鐘/個樣品：冷卻 20 分鐘

3. 40分鐘/個樣品：冷卻 30 分鐘

資料庫使用收費：

單獨使勇資料庫分析600/件。

NSYSU Department of Chemistry

Prof. Chun-Hu Chen’s Lab

X-ray Diffraction Instrument: using and charging rules

Begin from 8/1, 2015

Usage rules:

* The lowest initial scanning angle should be 2θ = 5°.

* Please including the cooling time when making reservation.

* Please make reservation at least one day earlier.

* One must write the User’s Record book after usage.

* If not operating properly, one will be forbidden to make reservation:

1. Solvents except water and alcohol are forbidden, or will be forbidden to make reservation for one week.

2. Opening the gate before turning off X-ray is consider as serious safety negligence, will be forbidden for two weeks.

3. Dropping the slit or not placing it properly, be forbidden for two weeks.

4. Above all, violating these rules over two times, one must take certification again.

Charging rules

| item | price (NT) | explanation |
| --- | --- | --- |
| Certification (for qualification of operate by self) | 1000 | Maximum certificated users per lab: 2 - teaching - practicing - test |

| item |  | Standard charging | explanation |
| --- | --- | --- | --- |
| Operate by self | In campus | 150/per sample | 1.standard parameter：2θ = 5° ~ 85°, increment=0.06, ~24 min，users could adjust depend on needs, within 29 min fits the standard charging. 2.If one wants to cost over 29 min/per sample, please see the “time out charge” explanation. 3.The charging including the usage of database. |
|  | Off campus | 250/per sample |  |
| Commission | In campus | 300/per sample | 1. standard parameter：2θ = 5° ~ 85°, increment=0.06, ~24 min，users could adjust depend on needs, within 29 min fit the standard charging. |
|  | Off campus | 600/per sample |  |

Time out charge：

- 30 min/per sample: cost 30~39 min/per sample, “standard charging” ×1.5 times

- 40 min/per sample: cost 40~49 min/per sample, “standard charging” ×2 times

- Over 50 min/per sample is not accepted.

Cooling rules:

1. 20 min/per sample: 10 min break

2. 30 min/per sample: 20 min break

3. 40 min/per sample: 30 min break

Please see Chinese version for complete information.